FOUNDATION CRACK OBSERVATION CHECKLIST

A repeatable observation and monitoring form for crack geometry, related movement, moisture, drainage, and escalation triggers.

| Property |  | Observation ID |  |
| --- | --- | --- | --- |
| Location |  | Date/time |  |
| Observer |  | Weather |  |
| Recent rainfall |  | Prior repair |  |

| LIMIT | This form records observations; it does not diagnose cause or establish structural safety. Rapid change, wall bowing, differential displacement, water entry near electrical systems, utility damage, or movement affecting occupancy warrants prompt qualified review. |
| --- | --- |

CRACK IDENTIFICATION

| Check | Observation / requirement | Notes / action |
| --- | --- | --- |
| [ ] | Mark the crack on a floor plan or elevation and assign a unique ID used in every photograph and measurement. |  |
| [ ] | Classify location: slab, foundation wall, basement wall, masonry veneer, interior finish, opening, or adjacent pavement. |  |
| [ ] | Classify orientation: vertical, horizontal, diagonal, stair-step, map cracking, or pattern associated with a joint/opening. |  |
| [ ] | Record length, maximum width, offset, depth/through-crack indication, leakage, staining, efflorescence, and prior patching. |  |
| [ ] | Photograph an overview and close-up with a ruler or crack comparator in the same plane as the crack. |  |
| [ ] | Record nearby floor slope, sticking doors/windows, separation, wall rotation, settlement, heave, or utility distress. |  |

MOISTURE, SOIL, AND SITE CONTEXT

| Check | Observation / requirement | Notes / action |
| --- | --- | --- |
| [ ] | Check grading, ponding, downspouts, gutters, irrigation, sump discharge, drains, leaks, and seasonal moisture changes. |  |
| [ ] | Record expansive clay indicators, desiccation cracks, erosion, soft ground, fill, trees, removed trees, or recent landscaping. |  |
| [ ] | Record nearby excavation, trenching, underpinning, additions, retaining walls, slopes, traffic vibration, or construction. |  |
| [ ] | Compare the crack with building age, construction joints, material changes, repair history, and known movement events. |  |

INITIAL CRACK LOG

| ID | Location | Pattern | Length | Width | Offset | Photo / notes |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |

| MEASURE CONSISTENTLY | Use the same marked points, instrument, units, viewing angle, and photograph scale at each visit. Record instrument resolution and environmental conditions so an apparent change can be distinguished from measurement variability. |
| --- | --- |

MONITORING RECORD

| Visit | Date | Crack ID | Width | Offset | Conditions | Photo / interpretation |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |

REVIEW TRIGGERS

| Check | Trigger | Action / responsible party |
| --- | --- | --- |
| [ ] | Width, offset, or length increases beyond measurement tolerance or the change accelerates. |  |
| [ ] | Horizontal cracking, wall bowing, rotation, differential displacement, or loss of support is observed. |  |
| [ ] | Cracking coincides with floor slope, sticking openings, utility distress, slope movement, or retaining-wall movement. |  |
| [ ] | Water entry, erosion, plumbing leakage, drainage failure, or soil-moisture change is active. |  |
| [ ] | The crack recurs after repair or appears in a structural element or location of uncertain construction. |  |
| [ ] | A qualified structural or geotechnical professional recommends survey, monitoring, exploration, drainage work, or repair design. |  |

OBSERVATION CONCLUSION

| Select | Disposition | Basis / next date |
| --- | --- | --- |
| [ ] | Document and recheck at the stated interval using the same location, method, and scale. |  |
| [ ] | Correct an identified drainage or leakage issue and continue monitoring. |  |
| [ ] | Request structural review, geotechnical review, survey, or other project-specific investigation. |  |
| [ ] | Restrict access or obtain urgent safety evaluation based on observed conditions. |  |

| Prepared by |  | Date |  |
| --- | --- | --- | --- |
| Reviewed by |  | Date |  |