GEOTECHNICAL REPORT REVIEW CHECKLIST

A structured review form for project basis, exploration, laboratory data, parameters, recommendations, constructability, and limitations.

| Project |  | Review no. |  |
| --- | --- | --- | --- |
| Report title |  | Report date |  |
| Consultant |  | Reviewer |  |
| Plans reviewed |  | Review date |  |

| REVIEW BASIS | Confirm that the geotechnical report and the design documents describe the same site layout, grades, loads, retaining systems, pavement areas, drainage concept, and construction stage. A technically sound report can be misapplied to a changed project. |
| --- | --- |

PROJECT DEFINITION AND SCOPE

| Check | Review criterion | Comment / action |
| --- | --- | --- |
| [ ] | Project location, proposed structures, grades, loads, walls, slopes, pavements, utilities, and earthwork are described consistently. |  |
| [ ] | The report states the purpose, scope, limitations, design stage, and documents used to plan the investigation. |  |
| [ ] | Changes since the investigation are identified and their effect on the recommendations is addressed. |  |
| [ ] | Geologic, geohazard, seismic, flood, karst, mining, landslide, expansive-soil, and contamination context is addressed where relevant. |  |

EXPLORATION AND FIELD DATA

| Check | Review criterion | Comment / action |
| --- | --- | --- |
| [ ] | Boring, test-pit, CPT, geophysical, or other exploration locations are shown relative to the proposed development. |  |
| [ ] | Exploration depths, termination criteria, refusal, drilling methods, sampling, recovery, and in situ tests are documented. |  |
| [ ] | Groundwater observations include timing, cave-in or stabilization limitations, seasonal qualification, and monitoring where needed. |  |
| [ ] | Logs describe fill, native soil, rock, weak layers, variability, odors/staining, loss zones, and conditions between explorations appropriately. |  |

LABORATORY TESTING AND INTERPRETATION

| Check | Review criterion | Comment / action |
| --- | --- | --- |
| [ ] | Index, classification, compaction, strength, consolidation, permeability, corrosion, and other tests match the design questions. |  |
| [ ] | Samples and test results can be traced to depth, material, specimen condition, stress level, and test method. |  |
| [ ] | Selected parameters distinguish total/effective stress, drained/undrained behavior, peak/residual strength, and short/long-term conditions. |  |
| [ ] | Parameter variability, correlations, data gaps, and engineering judgment are visible rather than hidden by a single value. |  |

FOUNDATIONS, SLABS, AND SETTLEMENT

| Check | Review criterion | Comment / action |
| --- | --- | --- |
| [ ] | Suitable bearing strata, allowable or factored resistance, footing dimensions, embedment, frost, groundwater, and excavation conditions are stated. |  |
| [ ] | Total and differential settlement basis, tolerable movement, load assumptions, and improvement alternatives are addressed. |  |
| [ ] | Deep-foundation recommendations address type, capacity, settlement, lateral behavior, installation, testing, and construction effects where relevant. |  |
| [ ] | Slab support, subgrade preparation, capillary break, vapor control, expansive soil, and floor-performance limitations are coordinated. |  |

EARTHWORK, PAVEMENTS, WALLS, SLOPES, AND DRAINAGE

| Check | Review criterion | Comment / action |
| --- | --- | --- |
| [ ] | Stripping, unsuitable materials, proof-rolling, subgrade stabilization, fill material, lift thickness, moisture, compaction, and testing are practical. |  |
| [ ] | Pavement inputs and sections identify traffic, subgrade, drainage, frost, stabilization, and maintenance assumptions. |  |
| [ ] | Wall parameters state movement condition, unit weight, strength, surcharge, water, seismic, passive resistance, global stability, and drainage assumptions. |  |
| [ ] | Slope and excavation recommendations address temporary/permanent geometry, groundwater, support, erosion, monitoring, and adjacent improvements. |  |

CONSTRUCTION AND REPORT LIMITATIONS

| Check | Review criterion | Comment / action |
| --- | --- | --- |
| [ ] | Construction observation, testing, acceptance, notification, and geotechnical review responsibilities are stated clearly. |  |
| [ ] | Temporary works, excavation safety, dewatering, vibration, underpinning, access, and sequencing risks are identified where relevant. |  |
| [ ] | Recommendations are coordinated with drawings/specifications and use terms that can be implemented and verified in the field. |  |
| [ ] | Report reliance, expiration/change conditions, variability, omitted services, and required updates are visible to the project team. |  |

REVIEW COMMENT REGISTER

| ID | Report section | Review comment | Priority | Owner | Disposition |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |
| 11 |  |  |  |  |  |
| 12 |  |  |  |  |  |
| 13 |  |  |  |  |  |
| 14 |  |  |  |  |  |
| 15 |  |  |  |  |  |

REVIEW CONCLUSION

| Select | Conclusion | Basis / action |
| --- | --- | --- |
| [ ] | Suitable for the stated design stage, subject to listed comments and project-specific coordination. |  |
| [ ] | Clarification or supplemental recommendations are required before affected design work proceeds. |  |
| [ ] | Additional exploration, testing, monitoring, or analysis is required. |  |
| [ ] | The report basis no longer matches the current project and should be updated by the geotechnical engineer of record. |  |

| Reviewed by |  | Date |  |
| --- | --- | --- | --- |
| Disposition by |  | Date |  |