SITE VISIT CHECKLIST

A field form for consistent observations, measurements, photographs, constraints, and follow-up actions.

| Project |  | Project no. |  |
| --- | --- | --- | --- |
| Location |  | Visit date/time |  |
| Observer(s) |  | Weather |  |
| Recent rainfall |  | Purpose |  |

| SAFETY | Complete a task-specific safety assessment before entering the site. Do not enter excavations, confined spaces, unstable slopes, traffic areas, or restricted property without appropriate authorization and controls. |
| --- | --- |

PRE-VISIT PREPARATION

| Check | Observation / requirement | Notes / action |
| --- | --- | --- |
| [ ] | Review available plans, prior reports, grading information, utility records, and the stated purpose of the visit. |  |
| [ ] | Confirm site access, owner or contractor contact, required personal protective equipment, and restricted areas. |  |
| [ ] | Prepare a marked base plan or aerial image for observation and photo locations. |  |
| [ ] | Confirm what decisions the visit is expected to support and what information cannot be established visually. |  |

GROUND SURFACE AND DRAINAGE

| Check | Observation / requirement | Notes / action |
| --- | --- | --- |
| [ ] | Record grades, low areas, ponding, swales, ditches, downspouts, erosion, sediment, and drainage outlets. |  |
| [ ] | Note seepage, springs, wet soil, staining, vegetation changes, sump discharge, irrigation, and utility-leak indicators. |  |
| [ ] | Trace where runoff comes from and where it leaves the observed area, including off-site contributions. |  |
| [ ] | Record recent grading, fill placement, excavation, trenching, or surface-cover changes. |  |

SOIL, ROCK, AND EARTHWORK OBSERVATIONS

| Check | Observation / requirement | Notes / action |
| --- | --- | --- |
| [ ] | Describe exposed material using visual-manual terms and distinguish apparent fill from native material where possible. |  |
| [ ] | Note soft, pumping, loose, organic, expansive, collapsible, erodible, or frost-susceptible conditions. |  |
| [ ] | Record rock outcrops, weathering, discontinuities, rippability observations, and excavation refusal if visible. |  |
| [ ] | Identify stockpiles, borrow sources, proof-roll response, compaction operations, and test locations where relevant. |  |

STRUCTURES, PAVEMENTS, WALLS, AND SLOPES

| Check | Observation / requirement | Notes / action |
| --- | --- | --- |
| [ ] | Document cracking, settlement, heave, rotation, lateral movement, distortion, leakage, and repair history. |  |
| [ ] | Check retaining-wall drainage, backfill condition, bulging, tilting, joint opening, toe condition, and crest loading. |  |
| [ ] | Check slopes for scarps, tension cracks, hummocky ground, toe bulging, erosion, seepage, and displaced features. |  |
| [ ] | Record pavement cracking, rutting, pumping, faulting, edge distress, base exposure, and drainage relationship. |  |

OBSERVATION AND MEASUREMENT LOG

| ID | Location | Observation | Measurement | Photo ID | Follow-up |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |

PHOTO LOG

| Photo | Location | Direction | Scale / reference | Description |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |
| 11 |  |  |  |  |
| 12 |  |  |  |  |

CLOSEOUT

| Check | Observation / requirement | Notes / action |
| --- | --- | --- |
| [ ] | Record areas that were inaccessible, covered, unsafe, or outside the authorized scope. |  |
| [ ] | Identify urgent safety or active-movement concerns communicated during or immediately after the visit. |  |
| [ ] | List additional exploration, survey, monitoring, testing, calculations, or specialist review needed. |  |
| [ ] | Confirm that photographs, notes, measurements, and marked plans are stored with the project record. |  |

| Prepared by |  | Date |  |
| --- | --- | --- | --- |
| Reviewed by |  | Date |  |