SLOPE STABILITY SCREENING CHECKLIST

A reconnaissance and data-needs form for identifying credible slope mechanisms, water controls, change, and consequence.

| Project |  | Slope ID |  |
| --- | --- | --- | --- |
| Location |  | Visit date |  |
| Observer(s) |  | Weather |  |
| Recent rainfall |  | Affected asset |  |

| STOP WORK | Active movement, falling rock, an unsupported excavation, damaged utilities, or distress affecting occupied structures or public access requires immediate site-specific safety evaluation. Do not rely on this checklist as a stability determination. |
| --- | --- |

GEOMETRY AND MATERIAL

| Check | Observation / requirement | Notes / action |
| --- | --- | --- |
| [ ] | Record slope height, inclination, length, orientation, benches, breaks in slope, and toe geometry. |  |
| [ ] | Identify apparent cut, fill, natural slope, embankment, waste fill, colluvium, residual soil, or rock slope. |  |
| [ ] | Note layering, weak seams, slickensides, contacts, discontinuities, weathering, and material variability. |  |
| [ ] | Estimate the plausible depth and shape of movement only if field evidence supports the interpretation. |  |

DISTRESS AND MOVEMENT INDICATORS

| Check | Observation / requirement | Notes / action |
| --- | --- | --- |
| [ ] | Map scarps, tension cracks, grabens, hummocky ground, toe bulging, lateral spreading, and displaced drainage features. |  |
| [ ] | Record tilted or curved trees, offset fences, cracked pavement, distorted walls, and damaged buried utilities. |  |
| [ ] | Distinguish erosion, shallow sloughing, rockfall, debris movement, translational movement, and rotational movement. |  |
| [ ] | Compare current observations with earlier photos, survey, monitoring, repair records, or witness reports. |  |

GROUNDWATER AND SURFACE WATER

| Check | Observation / requirement | Notes / action |
| --- | --- | --- |
| [ ] | Record seepage elevation, springs, wet zones, ponding, blocked drainage, erosion channels, and discharge points. |  |
| [ ] | Review rainfall, snowmelt, irrigation, septic discharge, water-line leakage, and stormwater changes. |  |
| [ ] | Note whether water is concentrated at the crest, within the slope, at contacts, or at the toe. |  |
| [ ] | Identify dry-season and wet-season conditions needed for a defensible stability model. |  |

LOADING AND GEOMETRY CHANGES

| Check | Observation / requirement | Notes / action |
| --- | --- | --- |
| [ ] | Record crest fill, stockpiles, structures, traffic, equipment, vibration, or other surcharge. |  |
| [ ] | Record toe excavation, erosion, channel migration, utility trenching, wall construction, or loss of support. |  |
| [ ] | Note vegetation removal, wildfire, grading, drainage work, and recent repair measures. |  |
| [ ] | Identify seismic, rapid drawdown, flood, wave, or construction stages relevant to the slope. |  |

ANALYSIS DATA NEEDS

| Check | Observation / requirement | Notes / action |
| --- | --- | --- |
| [ ] | Select candidate strength conditions: drained peak, drained residual, undrained, or rock-mass/discontinuity controlled. |  |
| [ ] | Identify exploration, sampling, laboratory testing, groundwater monitoring, and survey control required. |  |
| [ ] | Select plausible methods: infinite slope, circular/noncircular limit equilibrium, rock kinematics, numerical analysis, or monitoring only. |  |
| [ ] | Define sensitivity cases for groundwater, strength, loading, geometry, and credible adverse conditions. |  |

SLOPE GEOMETRY AND OBSERVATION LOG

| ID | Station / location | Feature | Dimensions | Photo / survey | Interpretation |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |

SCREENING DISPOSITION

| Select | Disposition | Basis / responsible party |
| --- | --- | --- |
| [ ] | No observed evidence of instability, but subsurface conditions and seasonal groundwater remain unverified. |  |
| [ ] | Additional reconnaissance, records review, or monitoring is required before selecting an analysis method. |  |
| [ ] | Detailed geotechnical exploration and stability analysis is recommended. |  |
| [ ] | Urgent safety, access, drainage, or stabilization measures require project-specific professional direction. |  |

| Prepared by |  | Date |  |
| --- | --- | --- | --- |
| Reviewed by |  | Date |  |